- The use of the external module Ab_An.dll

Ab_An.dll file is an external module for the Gibbs program, and is designed to calculate the activity coefficients of the components of albite-anortite solid solution. Using this module in the calculation of equilibria via HCh package programs provides better, more realistic results than when running the programs without it.

A non-ideal model of albite-anortite solid solution, proposed in literature [1], is used in this module.

#### Installation

In order to make the module available for the HCh package, it is enough to place it in the home directory of the package (normally, this is C:\HCh_Win\Main).

#### Preparation of the System file

To be able to use the module Ab_An.dll, you need to include a binary solid solution of albite and anortite into the System file, as you create it. These minerals are included into the default database Unitherm, a part of the HCh package; it is also possible to use tailored description of these minerals. If you use a tailored description, you need to name these minerals in such a way, which will allow the module to recognize them. Note that the module recognizes end-members of the solution by name, or rather by the first three letters of their names: the name of the albite component should start with the letters "alb", the name of the anortite component – with "ano" (both case insensitive). You need to use the same approach when renaming the standard database components. After creating a solid solution as described above, you can assign any name to it (for example, "Plagioclase"); the only important thing is to make sure that the name is unique within the System file.

After creating the System file, you must add a text file named Modules.txt, containing the line "<module name>: <name of the solution>" (in our example it would be "Ab_An: Plagioclase" – do not use the quotes) to the project folder. The file Modules.txt is needed for determining which solutions of the current project require processing by external modules, and which modules need to be used. If the file Modules.txt already exists in the project folder, then simply add to its contents the line, specified above. Be careful here: the name of your solution must be written exactly as it appears in the System file (here registers are important!). If the exact name of your solution is not found in the file Modules.txt, the Gibbs program will ignore the external module and the error notification will not be displayed.

##### Computations with an external module

To allow the external modules, listed in the file Modules.txt, to dynamically link to the Gibbs program during the calculations, you need to run the program with the option "Attach external modules" activated. If the Gibbs program is running in the interactive mode (i.e. with the source files such as Blank or Input), this option is set in the Main program through the top menu: Gibbs – Gibbs Options – Computing… – Attach external modules. Similarly, it is set for the Control file when editing the list of Gibbs program options. If the program Gibbs is invoked from a user’s application (see GibbsLib.doc), then this option should be set in the property GibbsOptions by adding the parameter "/em" ("External modules").

##### The results of using the external module

When an external module is dynamically linked to the Gibbs program, the activity coefficients of the components of the corresponding solution are calculated using this module. The results of these calculations are easiest to see in the Gibbs program listing, in the case when the program is run in the interactive mode (with a Blank or Input file). In this case, the program provides a full listing, which includes the activity coefficients. Note that in the complete listing the name of the external module, used in the calculation, is always listed under the row with the name of the solution.

When the Gibbs program is run in the automatic mode (with a Control file), the values ​​of the activity coefficients are never shown, so in this case the effect of the external module can only be estimated indirectly.

If the values of the activity coefficients, calculated by the module, are needed, for example, in the case when the Gibbs program is invoked from any user's application, this information can be obtained by using the parameter lna of the CallGibbs method or the property GibbsResult_g of the object Gibbs (see GibbsLib.doc).

A use of the external module Ab_An.dll was demonstrated in [2].

#### Error messages

If the program Gibbs detects an error when an external module is added or accessed, it displays the error notification and terminates. Of course, errors may occur due to incorrect operation of the Gibbs program itself but most of the time the errors are caused by the actions of the user. The list of errors detected by the Gibbs program can be found in the file GibbsExt.doc; here we show only the notifications of errors, which are detected by the Ab_An.dll only. All of these errors are detected during the initial call to the module (Primary call) and are characterized by the value of the numeric code:

| Code | Description | Comment |
| --- | --- | --- |
| -1 | Too many solutions | There are several solutions that are assigned to the current module in the System file, which makes no sense. Delete the excess solid solutions from the System file. |
| 1 | Non-binary solution | The solution, used by the module, consists of more than two minerals, which is not allowed. Correct the System file. |
| 2 | Incorrect mineral name | The name of one of minerals must begin with "alb", and the other – with "ano". Check the System file. |

#### References

- Holland T.J.B., Powell R. Plagioclase feldspars: activity-composition relations based upon Darken’s quadratic formalism and Landau theory // Amer. Mineral., 1992, v. 77, p. 53-61.

- Shvarov Yu.V. HCh: New potentialities for the thermodynamic simulation of geochemical systems offered by Windows // Geochemistry International, 2008, v. 46, # 8, p. 834-839.